Q. Demonstrate the Static Routing in Packet Tracer.

Aim : To write a code to demonstrate static routing in packet tracer.

Steps:

- In packet tracer take 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

- Connect both router via fastethernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Code:

Router0:

Router> enable

Router# configure terminal

Router(config)# interface fastethernet0/0

Router(config-if)# ip address 10.0.0.1 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1

Router(config-if)# ip address 20.0.0.1 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)#ip route 40.0.0.0 255.0.0.0 20.0.0.2  Router(config)#exit Router1:

Router> enable

Router# configure terminal

Router(config)# interface fastethernet0/0

Router(config-if)# ip address 20.0.0.2 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1

Router(config-if)# ip address 40.0.0.1 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)#ip route 10.0.0.0 255.0.0.0 20.0.0.1

Router(config)#exit

Verification: In Both  PCs Command Prompt.

PC0:

C:\ipconfig

C:\ping 10.0.0.1

C:\ping 10.0.0.2

C:\ping 20.0.0.1

C:\ping 20.0.0.2

C:\ping 40.0.0.1

C\:ping 40.0.0.2

PC1:

C:\ipconfig

C:ping 40.0.0.1

C:ping40.0.0.2

C:ping 20.0.0.2

C:ping 20.0.0.1

C:ping 10.0.0.1

C:ping 10.0.0.2

Output:

PC0:

PC1:

Conclusion:The above Topology has been executed successfully.