Study of Networking Device

Q1.  Identify the  network address & subnet mask for the ip address 192.168.1.0

Given ip address is 192.168.1.0

The subnet mask is 255.255.255.0  as it is belong to class C

.In Binary Form:-

11111111 .11111111 .11111111.00000000 In this the 1st 24 bits represent host part.

Q2. Create 4 Subnets:-

To create 4 subnets we need to binary two bits from the host part.

2^2=4

So the new subnet mask is  255.255.255.192

Binary Form:-

11111111.11111111.11111111.11000000

so  26 bits for the network part 6 bits for host part. Total no. of subnet 4.

[ combining 2 or more subnet will give us the Supernet.]

Total no. Of useable ip address in each subnet is 64- -2 because  1 address is for network and 1 for broadcast, so subtracting them.  62 IP address are useable.

CIDR format:-classless InterDomain Representaion.

New Subnet Mask:- 255.255.255.192/126

The 1st subnet mask is 192.168.1.0/126

The  2nd subnet mask is 192.168.1.64/126

Subnet 1=192.168.1.0 ---> Network address

Useable host address 192.168.1.62/126

Broadcast address 192.168.1.63/26

Subnet 2 192.168.1.64

1st  host address 192.168.1.65

Last host address                     192.168.1.126/26

Broadcast address                  192.168.1.127

Subnet 3                                        192.168.1.128

Last address is                           192.168.1.190

15th address is                           192.168.1.129

Subnet 4                                        192.168.1.192

Last address is                            192.168.1.254

Broadcast address                     192.168.1.255

Q3. A network address     192.168.1.0/24  create 3 subnet with subnet 1 corristing if so subnet 2 with 20, and subnet 3 with 10.

(Since the no. Of host bit is determine of the ip address. So ip address formula to calculate no. Of bits required to satisfy  each subnet ip address is 2^n/   no.  of ip address required.

Where n is the no. of bits and  +2 represent n|6)

Subnet 1 of  50 ip address

Ex:- 2^6 = 64    with 62 useable ip address we need 6 host bits.

Subnet mask is 255.255.255.192/26

2^5  = 32 ,30 useable

Subnet mask is 255.255.255.224|27

2^4=16

Subnet   255.255.255.140|28

Subnet 1

Subnet  mask:                                                     255.255.255.192|26

Network address                                                192.168.1.0|26

Host address                                            192.168.1.1|26

Last host address                                    192.168.1.50|26

Useable ip address                           62//

Broadcast                                              192.168.1.63|26

Q. Demonstrate the Static Routing in Packet Tracer.

Aim : To write a code to demonstrate static routing in packet tracer.

Steps:

- In packet tracer take 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

- Connect both router via fastethernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Code:

Router0:

Router> enable

Router# configure terminal

Router(config)# interface fastethernet0/0

Router(config-if)# ip address 10.0.0.1 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1

Router(config-if)# ip address 20.0.0.1 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)#ip route 40.0.0.0 255.0.0.0 20.0.0.2  Router(config)#exit Router1:

Router> enable

Router# configure terminal

Router(config)# interface fastethernet0/0

Router(config-if)# ip address 20.0.0.2 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1

Router(config-if)# ip address 40.0.0.1 255.0.0.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)#ip route 10.0.0.0 255.0.0.0 20.0.0.1

Router(config)#exit

Verification: In Both  PCs Command Prompt.

PC0:

C:\ipconfig

C:\ping 10.0.0.1

C:\ping 10.0.0.2

C:\ping 20.0.0.1

C:\ping 20.0.0.2

C:\ping 40.0.0.1

C\:ping 40.0.0.2

PC1:

C:\ipconfig

C:ping 40.0.0.1

C:ping40.0.0.2

C:ping 20.0.0.2

C:ping 20.0.0.1

C:ping 10.0.0.1

C:ping 10.0.0.2

Output:

PC0:

PC1:

Conclusion:The above Topology has been executed successfully.

Q. Demonstrate Routing OSPF in packet tracer.

Aim: To write a code to demonstrate routing OSPF in packet tracer.

Steps:

- In packet tracer take 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

- Connect both router via fastethernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Code:

Router0:

Router> enable

Router# configure terminal

Router(config)# interface fastethernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1

Router(config-if)# ip address 10.0.0.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)#router ospf 1

Router(config-router)#network 192.168.1.0 0.0.0.255 area 0

Router(config-router)#exit

Router1:

Router> enable

Router# configure terminal

Router(config)# interface fastethernet0/0

Router(config-if)# ip address 10.0.0.2 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fastethernet0/1

Router(config-if)# ip address  192.168.2.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)#router ospf 2

Router(config-router)#network 10.0.0.0 0.0.0.255 area 0

Router(config-router)#network 192.168.2.0 0.0.0.255 area 0

Router(config-router)#exit

Verification :In PC in command Prompt.

PC0:

C:\ipconfig

C:\ping 192.168.2.10 Output:

Verification: On any one Router

Router#show ip ospf neighbor

Router#show ip protocol

Router#show ip protocol Output:

Conclusion : The above Topology has been executed successfully.

Q. Demonstrate the RIP Routing in Packet Tracer.

Aim : To write a code to demonstrate RIP routing in packet tracer.

Steps:

- In packet tracer take 2 PCs and 2 Routers.

- Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

- Connect both router via fastethernet0/1 using cross-copper cord.

- Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Router0:

Router> enable

Router# configure terminal

Router(config)# interface fastethernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1

Router(config-if)#  ip address 10.0.0.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 192.168.1.0

Router(config-router)# network 10.0.0.0

Router(config-router)# no auto-summary

Router(config-router)# exit

Router(config)# end

Router1:

Router> enable

Router# configure terminal

Router(config)# interface gigabitEthernet0/0

Router(config-if)# ip address 10.0.0.2 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1

Router(config-if)# ip address 192.168.2.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# router rip

Router(config-router)# version 2

Router(config-router)# network 10.0.0.0

Router(config-router)# network 192.168.2.0

Router(config-router)# no auto-summary

Router(config-router)# exit

Router(config)# end

Router#show ip route Router#show ip protocol

Output:

Verification:In PC0 command prompt.

C:>ipconfig

C:\>ping 192.168.2.2 Output:

Conclusion:The abovee topology has been executed successfully. Q.

Q.Demonstrate DHCP router in packet Tracer.

Aim:- To demonstrate DHCP server in packet Tracer.

Setup:

(1). Router , Switch & 2 Pcs.

(2).Connect the Pcs to switch and connect switch to the router.

(3).Give router a ip address then in Pc0 & Pc1 click the radio button of DHCP in the ip configureration panel the values will be assingned directly.

Topology:

Code:

Router> enable

Router# configure terminal

Router(config)#

Router(config)# interface fastEthernet0/0

Router(config-if)# ip address 192.168.10.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# ip dhcp pool MYPOOL

Router(dhcp-config)# network 192.168.10.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.10.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

Verification: In Both  PCs Command Prompt.

PC0:

C:\ipconfig

C:\>ping 192.168.10.1

Output:-

Conclusion:The above Topology has been executed successfully.

Q. Demonstrate DNS server in packet Tracer.

Aim:- To demonstrate DNS server in packet Tracer.

Setup:

(1). One Pcs,One  Switch & One server.

(2). Connect Pcs to switch and switch to server.

(3).Then in server on the DNS services.

(4). Then origin ip address and DNS server .

(5). For Pc also origin ip address and DNS server ip of the server.

Topology:

Config:

(1).Go to HTTP  in server.

(2). There in file name index make some changes.

(3). Save it and then in DNS service add the name for the website and the DNS server and add  that detail.

(4). Now go to Pc in there desktop click web browser and put the name of the file in URL and click go.

(5). If the topology work successfully then the website will be displayed.

Verification: In   PCs Command Prompt.

PC0:

C:\ipconfig

C:\ping www.mywebsite.com

Output:

Conclusion:The above Topology has been executed successfully.

Q.  Demonstrate FTP server in packet Tracer.

Aim:-To demonstrate FTP server in packet tracer.

Setup:

(1). 2  Pc , 1 switch,  1 server.

(2).Connect  Pcs to switch and switch to server.

Topology:

Config: PC0

assign IP address           192.168.1.10

Subnet Mask                  255.255.255.0

PC1

assign IP address               192.168.1.11

Subnet Mask                  255.255.255.0 Server:

(1). Enable FTP option.

(2).Create username and password(123).

(3). Click all the  5 tick bores  write , read ,Delete ,Rename list.

(4). Add it.

PC0:

C:\ipconfig

C:\>ping 192.168.1.2 C:\>ftp 192.168.1.2

ftp>dir

Output:

Conclusion:The above Topology has been executed successfully.

Q. Demonstrate HTTP server in Packet tracer.

Aim:- To demonstrate HTTP server in packet Tracer.

Setup:

(1).  1 Pc  ,1 switch ,2 Server.

(2). Connect Pc with switch and switch with the servers.

(3).Connect Router0 with PC0 via fastethernet0/0 using copper stright wire and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

(4).Connect both router via fastethernet0/1 using cross-copper cord.

(5).Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Config:

Server  0 (DNS Server)

Assign ip address                       192.168.1.1

DNS  Server                                           192.168.1.1

Server 1 (web server)

Assign   ip address    192.168.1.2

DNS Server         192.168.1.1

DNS  Server will be the same

PC0

Assign  ip address   192.168.1.3

DNS   Server            192.168.1.1

(1). In DNS server 0 select and on DNS .

(2).  Then go to web server   http and edit the index.

(3). Name the website in DNS  column and add the web server address and make on entity i the below list by  clicking add.

(4). In PC0 open the desktop  and then web browser  write the name of the website if the edited html files contet displays ,then the topology has been succcessfully established.

PC0:

C:\ipconfig

C:\>ping 192.168.1.3

Output:

Conclusion:The above Topology has been executed successfully.

Q. Demonstrate telnet in packet Tracer.

Aim:-To demonstrate Telnet in packet Tracer.

Setup:

(1). 1 PC , 1 Router , 1 switch.

(2). PC --->Switch--> Router  connect using copper straight wire through cables.

(3). Do Router ,Switch interface  Up.

Topology:

Config:

Switch :

Switch>enable

Switch#configure terminal

Switch(config-if)# ip address) 192.168.1.2  255.255.255.0

Switch(config-if)# no shutdown

Switch(config-if)#exit

Switch(config-if)# line vtg 04

Switch(config-if)# password cisco

Switch(config-line)#login

Switch(config-line)#end

Switch# write memory Router:

Router>enable

Router#configure terminal

Router(config-if)# ip address 192.168.1.1

Router(config-if)#no shutdown

Router(config-if)#exit

Router(config)# line vty 04

Router(config-line)# password cisco

Router(config-line)#login

Router(config-line)#end

Router# write memory

Verify:    PC0:

C:\ipconfig

C:\>ping 192.168.1.1

Telnet 192.168.1.1

Telnet 192.168.1.2

Output:

Conclusion:The above Topology has been executed successfully.

Study of cryptography substitution Techniques.

Q. Demonstrate  substitution technique in C.

Aim: To demonstrate substitution Technique in C.

Program:-  #include <stdio.h> #include <conio.h>  void main()

{

char str [26];

int i; clrscr(); printf("Enter plain text: ");  scanf("%s",& str);

for (i=0; str[i]:= '\0'; i++)

{

str[i] = str[i]+4;  printf("Encrypted text: +4 In");  printf( Encrypted : %5 .s\n", str);  for (i=0; str[i]:= 10;i++)

{

str[i] = str[i]-4;

}

printf(“Decrypted text:-4 \n”); printf(Decrypted:%>\n,str); getch();

}

Result:

Enter plain text:HELLO

Encrypted text:+4

Encrypted:KHOOR Decrypted:-4

Decrypted: HELLO

Conclusion:The above program has been executed successfully.

Study of Symmetric Key.

Q.Demonstrate AES in Python.

Aim:-To write a python program to demonstrate AES in python.

Program:-

! Pip  install pycryptodome from crypto.cipher import AES

from Crypto.Util.Padding import pad import binascii  sykey=bytes.fromhex('2b7e151628aeb2a6abf7158809cf4f3c') originaltext=b"0123456789ABCDEF" ciphertext=AES.new(sykey,AES.MODE_ECB) cencrypt=ciphertext.encrypt(originaltext) print(cencrypt.hex) ciphertext1=cencrypt.hex() ctext=AES.new(sykey,AES.MODE_ECB) btext=ctext.decrypt()  Result:

Conclusion: The above program has been executed successfully.

Study of Asymmetric key.

Q. Demonstrate RSA in python.

Aim: To write s program in python to demonstrate RSA.

Program:

p=5 q=11 n=p*q pn=(p-1)*(q-1) e=3 d=27 m=9 a=pow(m,e,n) print("Cipher",a) decryption=pow(a,d,n)

print("decrypted",decryption)

Result:

Conclusion:The above program has been executed successfully.

Study of MDS Algorithm

Q. Demontrate MDS algorithm in python.

Aim : To write a program to demonstrate MDS Algorithm in python.

Program:

import hashlib str1="syit class" md5hashvalue=hashlib.md5() print(md5hashvalue) md5hashvalue.update(str1.encode('utf-8')) md5hashvalue messagedigest=md5hashvalue.hexdigest() messagedigest print(messagedigest)

Result:

Conclusion: The above program has been executed successfully.

Study of Mash Function RSHash

Q.Demonstrate RSHash Function in python.

Aim: To write a python program to demonstrate RSHash function.

Program:

def rshashvalue(str2):

a=63689   b=378551   hashvalue=0   for ch in str2:

hashvalue=(hashvalue*a+ord(ch))     a=a*b     return hashvalue & 0xFFFFFFFF h1=rshashvalue('syit class')

Print(h1)

Result:

Conclusion: The above program has been executed successfully.